Specification

Table 2

| Parameter |  | Value |  |  |  |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supply voltage |  |
| Supply voltage , V |  | 9 ÷ 36 |  |  |  |  |  |  |
| Current consumption at 24 V with out data transmission, mA |  | 45 |  |  |  |  |  |  |
| Current consumption at 24 V with data transmission, mA |  | Up to 100 |  |  |  |  |  |  |
| Autonomous operation time |  | Up to 4 h |  |  |  |  |  |  |
| Operational conditions |  |
| Operational temperature conditions |  |  |  |  |  |  |  |  |
| With data transmission, °С |  | -40 ÷ +85 |  |  |  |  |  |  |
| On a backup battery, °С |  | -20 ÷ +65 |  |  |  |  |  |  |
| Internal battery charging , °С |  | 0 ÷ +45 |  |  |  |  |  |  |
| Ingress protection |  | IP51 / IP54 |  |  |  |  | No connectors’ lid/ Connectors’ lid and cable glands are used |  |
| Size and mass |  |
| Weight, g, not more |  | 450 |  |  |  |  |  |  |
| Internal sensors and modules |  |
|  |  | PRO/PRO 285 |  | ENT/STD/Lite |  |  |  |  |
| Internal accelerometer |  | + |  |  |  |  |  |  |
| Tamper switch |  | + |  |  |  |  |  |  |
| Traffic encryption |  | + |  |  |  |  |  |  |
| Built in Flash, Mb |  | 16 |  | 4 |  |  |  |  |
| Historical playback entries |  | 500 000 |  | 350 000 |  |  |  |  |
| Ports and interfaces |  |
|  |  | PRO/PRO 285 | ENT |  | STD | Lite |  |  |
| Universal inputs, amount |  | 6 | 6 |  | 6 | 1 |  |  |
| Output (OC), amount |  | 2 | 2 |  | 2 | 1 |  |  |
| RS-485 Port | ScoutNet | + | + |  | + | + |  |  |
|  | NMEA | + | - |  | - | - |  |  |
|  | LLS | + | +* |  | + | + |  |  |
|  | J1708 | + | +* |  | - | - |  |  |
|  | RFID | + | +* |  | - | - |  |  |
| RS-232 Port | LLS | + | - |  | - | - |  |  |
|  | CAN-Log | + | - |  | - | - |  |  |
|  | VDO Tachograph | + | - |  | - | - |  |  |
|  | NMEA | + | - |  | - | - |  |  |
| CAN (J1939, Mobileye, OBD) |  | + | + |  | - | - |  |  |
| USB port |  | 1 | - |  | - | - |  |  |

(*)MT-700 ENT contains two RS 485 ports, one for ScoutNET protocol and the other – for LLS(or J1708,RFID) protocol.