ScoutOpen 2 protocol

Protocol description

General information

Terminal ID must be pre-registered on the server. The terminal initiates an outgoing connection to the server to the IP address and port specified in the settings.

Byte order – little-endian.

Standard types:

uint8_t, uint16_t, uint32_t, uint64_t non-integer data type with the size of 1, 2, 4, 8 bytes, respectively.

int8_t, int16_t, int32_t, int64_t – integer data type with the size 1, 2, 4, 8 bytes, respectively.

float – a number with a single-accuracy floating point corresponding to the IEEE 754 standard, size 4 bytes.

Dump[] – bytes array.

String – text in the Windows-1251 encoding, the first byte is the length of the string. Example: String "321" will be passed by a byte sequence {0x03, 0x33, 0x32, 0x31}.

Transmission order

- The terminal opens the connection to the server.

- The terminal transmits a frame containing one or more packets with coordinates, time and sensor readings.

- The server responds with 0x55, otherwise the transmission is repeated.

- If there is more data, then the return to step 2, otherwise the transmission stops.

Frame format

| Title | Type | Size | Description |
| --- | --- | --- | --- |
| FrameSize | uint16_t | 2 bytes | Frame size |
| NumberOfPackets | uint16_t | 2 bytes | Number of packets per frame (N) |
| Packet 1 | PacketType | n1 bytes | Packet 1 |
| Packet 2 | PacketType | n2 bytes | Packet 2 |
| … |  |  |  |
| Packet N | PacketType | nN bytes | Packet N |
| CRC8 | uint8_t | 1 byte | Check sum |

FrameSize – the full frame size in bytes.

NumberOfPackets – the number of packets in the frame.

CRC8  –  - checksum of the frame.  Calculated according to CRC8 algorithm using x^8 + x^7 + x^4 + x^0 forming polynomial. The checksum is counted over all bytes of the frame, except the frame length field and the checksum field itself.

Telemetry data packet format

| Title | Type | Size | Description |
| --- | --- | --- | --- |
| Service information |  |  |  |
| Size | uint16_t | 2 bytes | Packet size |
| PacketType | uint16_t | 2 bytes | Packet type |
| DeviceType | uint32_t | 4 bytes | Equipment type |
| Id | String | n- bytes | Identifier |
| General data |  |  |  |
| DateTime | DateTime | 8 bytes | Time in .NET format |
| Longitude | float | 4 bytes | Longitude, degrees |
| Latitude | float | 4 bytes | Latitude, degrees |
| Speed | float | 4 bytes | Speed, km/h |
| Course | uint16_t | 2 bytes | Course, degrees |
| Satellites | uint8_t | 1 byte | Number of satellites in use |
| Fix | uint8_t | 1 byte | Signs of a fixation of coordinates |
| Odometer | uint32_t | 4 bytes | Mileage, m |
| Status | uint16_t | 2 bytes | Terminal status |
| Sensor data |  |  |  |
| Sensor 1 | SensorData | n- bytes | Sensor 1 data |
| Sensor 2 | SensorData | n- bytes | Sensor 2 data |
| … | … | … | … |
| Sensor N | SensorData | n- bytes | Sensor N data |

Service information and general data in the packet must be available and there may be no sensor data in the package. Availability of these sensors and their size is based on the total size of the package.

Size – the total size of the packet in bytes.

PacketType – Packet type. For telemetry data packet = 1.

Id – the factory ID of the terminal (serial number). The first byte is the length of the string (not including the first byte).

DeviceType – terminal type.

DateTime – time in .NET format. One clock corresponds to 100 nanoseconds or one ten millionth of a second. The value of this property represents the number of 100 nanosecond intervals that have passed since midnight 00:00:00:00, January 1, 0001, which corresponds to the value of DateTime.MinValue.

Time conversion in Unix-time

Fix – coordinate fixation sign

| Bit | Value |
| --- | --- |
| 0 | 1 – coordinates are fixed 0 – no fixation of coordinates |
| 1 - 7 | Reserve |

Status – terminal status

| Bit | Value |
| --- | --- |
| 0 | Power: 1 – main 0 – backup |
| 1 | Movement: 1 – movement 0 - parking |
| 2 | Ignition 1 – Ignition on 0 – Ignition off |
| 15 | The validity of the Status field 1 – Status field is valid 0 – not valid (the "Status" field should not be used) |
| 3 - 14 | Reserve |

Sensor data format

SensorData – sensor data

| Title | Type | Size | Description |
| --- | --- | --- | --- |
| PortNumber | uint8_t | 1 byte | Port number |
| SensorNumber | uint8_t | 1 byte | Sensor number at the port |
| SensorType | uint8_t | 1 byte | Sensor type |
| DataSize | uint8_t | 1 byte | Sensor data size |
| Data | SensorType | DataSize byte | Sensor data |

The numbering of ports and sensors (PortNumber, SensorNumber) starts from scratch.

SensorType – sensor type.

Sensor types

| Code | Title | Type | Description |
| --- | --- | --- | --- |
| 1 | analog | float | Used to transmit the current value of the analog sensor (example: voltage or fuel level) |
| 2 | counting | uint | Used to transmit value changes, must be summed up on the server (example: flowmeter) |
| 3 | discrete | uint8_t | Used to transmit a discrete state (example: ignition on/off), takes on 0 ... 1 |
| 4 | binary | Dump[] | Used to transfer an array or bit field (example: error word) |
| 5 | Linear acceleration | int16_t | Linear acceleration, mg. Positive value corresponds to forward acceleration, negative value corresponds to deceleration (backward acceleration). |
| 6 | Side acceleration | int16_t | Side acceleration, mg. Positive value corresponds to the direction to the right (turn right), negative value corresponds to the acceleration to the left. |
| 7 | Traffic accident sensor | Dump[] | Contains the information on operation of the sensor of road accident. |
| 8 | Identification of GSM BS | Dump[] | Contains MCC/MNC, LAC, Cell ID of the base station. |
| 9 | Acceleration threshold exceeded | Dump[] | Contains the maximum and average acceleration values for the time of exceeding and duration of exceeding. |
| 10 | Additional navigation data | Dump[] | Contains altitude and parameters for deterioration of navigation accuracy. |
| 11 | IMEI | String | Text string containing modem IMEI. |
| 12 | Identification | Dump[] | Identification data. |

Linear acceleration

Linear acceleration, units of measure - mg. Positive value corresponds to forward acceleration, negative value corresponds to deceleration (backward acceleration). Signal, 2 bytes (int16_t).

Side acceleration

Side acceleration, units of measure - mg. The positive value corresponds to the right direction (turn right), the negative value corresponds to the left acceleration. Signal, 2 bytes (int16_t).

Accident sensor

| Type | Data |
| --- | --- |
| uint32_t | CrashID – unique accident identifier |
| uint64_t | DateTime - time in the .NET format |

Identification of GSM base station

| Type | Data |
| --- | --- |
| uint32_t | MCC/MNC – Country code/cellular network code |
| uint32_t | LAC – local area code |
| uint32_t | Cell ID – cell identifier |

Acceleration threshold exceeded

| Type | Data |
| --- | --- |
| uint8_t | AccelType- the type of acceleration of threshold excess 0 – linear (longitudinal) 1 – lateral (transversal) |
| int32_t | AccelerationMax - the maximum acceleration value for the excess time, [mg]. Positive value is acceleration, negative value is braking. |
| int32_t | AccelerationAverage – is the average acceleration value for the excess time, [mg]. Positive value is acceleration, negative value is braking. |
| uint32_t | ExceedingTime – the duration of the acceleration excess, [ms]. |

Additional navigation data

| Type | Data |
| --- | --- |
| int16_t | Altitude - altitude, [m] |
| unt16_t | VDOP – reduction of accuracy in the vertical plane * 0.01 |
| unt16_t | HDOP – reduction of accuracy in the horizontal plane * 0.01 |
| unt16_t | PDOP – reduction of positioning accuracy * 0.01 |

Identification

| Type | Data |
| --- | --- |
| uint8_t | Type of identification: 0 – Driver identification 1 – Identification on the refueler 2 – identification of attachments 3 ..255 – Reserve |
| uint8_t | Identifier length, n |
| Dump[n] | Identifier, lower byte forward |

Example: 0x00 0x07 0xA0 0xBD 0x05 0xF7 0x32 0x58 0x91
0x00 – driver identification
0x07 – identifier length 7 bytes
0xA0 0xBD 0x05 0xF7 0x32 0x58 0x91 – identifier 915832F705BDA0

Traffic accident data packet format

The package is designed to transmit the acceleration array along the 3-axis fixed by the sensor accident.

The array is transmitted in blocks in several frames, the sign of the last data block Completed = 1.

| Name | Type | Size | Description |
| --- | --- | --- | --- |
| Service information |  |  |  |
| Size | uint16_t | 2 bytes | Packet size |
| PacketType | uint16_t | 2 bytes | Packet type |
| DeviceType | uint32_t | 4 bytes | Equipment type |
| Id | String | n- bytes | Identifier |
| Traffic accident data |  |  |  |
| DateTime | DateTime | 8 bytes | Time in .NET format |
| CrashID | uint32_t | 4 bytes | Unique accident identifier |
| Completed | uint8_t | 1 byte | 1 – last block of traffic accident data |
| Offset | uint16_t | 2 bytes | Accelerator block offset from array start, byte |
| DataSize | uint16_t | 2 bytes | Acceleration block size, byte |
| Acceleration[] | Dump[DataSize] | DataSize byte | Array of accelerations during accident |

Size – the full size of the packet in bytes.

PacketType – Type of packet. For accident data packet = 2.

Id – the factory ID of the terminal (serial number). The first byte is the length of the line (not including the first byte).

DeviceType – the type of terminal.

DateTime – time in the .NET format.

CrashID – a unique accident identifier.

Completed – sign of the last traffic accident data packet (1 - data transmission completed, 0 - transmission not completed).

Offset – offset of data from the beginning of the array.

DataSize – the size of the data block of the accident in the package.

Acceleration[] – array of accelerations during accident.

| Acceleration[0] | int16_t | X-axis acceleration, mg |
| --- | --- | --- |
|  | int16_t | Y-axis acceleration, mg |
|  | int16_t | Z-axis acceleration, mg |
| Acceleration[1] | int16_t | X-axis acceleration, mg |
|  | int16_t | Y-axis acceleration, mg |
|  | int16_t | Z-axis acceleration, mg |
| … | … | … |
| Acceleration[DataSize / 6 - 1] | int16_t | X-axis acceleration, mg |
|  | int16_t | Y-axis acceleration, mg |
|  | int16_t | Z-axis acceleration, mg |

2. Transfer example

Terminal: 4e 00 01 00 49 00 01 00 19 00 00 00 05 33 30 31 31 32 00 4b 71 00 e7 bc cd 08 2b 98 0a 42 89 06 66 42 33 33 97 42 41 01 0f 01 d6 d0 00 00 07 80 00 00 01 04 e7 0c c3 41 01 00 03 01 00 0a 00 01 04 00 00 bf 42 0a 01 02 04 32 00 00 00 3A

Server: 55 // confirmation of reception

Frame content

Frame header

4e 00 – the size of the frame: 78 bytes

01 00 – number of packets in the frame: 1

The packet:

Service information:

49 00 – packet size: 73 bytes

01 00 – Package type: 1 - Telemetry data

19 00 00 00 – equipment type: 25

05 – ID line length: 5 bytes

33 30 31 31 32 – ID: 30112

00 4b 71 00 e7 bc cd 08 – time in the .NET format: 30.03.2011 11:44:46

2b 98 0a 42 – longitude: 34.648602

89 06 66 42 – latitude: 57.506382

33 33 97 42 – Speed: 75.6 km/h

41 01 – course: 321

0f – number of satellites: 15

01 – Fixation of coordinates: Fixed

d6 d0 00 00 – Mileage: 53462 m

07 80 – Terminal status: main power, movement, ignition on, field valid

Sensors data:

00 – port: 0

00 – sensor: 0

01 – data type, analog

04 – data size: 4 bytes

e7 0c c3 41 – sensor reading: 24.3813

01 – port: 1

00 – sensor: 0

03 – data type, discrete

01 – data size: 1 byte

00 – sensor reading: off

0a – port: 10

00 – sensor: 0

01 – data type, analog

04 – data size: 4 bytes

00 00 bf 42 – sensor reading: 95.5

0a – port: 10

01 – sensor: 1

02 – data type, counter

04 – data size: 4 bytes

32 00 00 00 – sensor reading: 50

The checksum of the frame:

3A – CRC8